T1175 Power Delivery Firmware Update Process

- Requirements

  - USB Flash Drive (1GB minimum) – may be provided by MobileDemand upon request

  - USB Keyboard

    - May need a USB extender for multiple USB-A devices (Flash Drive and Keyboard)

  - Power Adapter

POWER IS REQUIRED FOR ALL BELOW PROCESS

- Go to the MobileDemand website

  - Open your internet browser

  - Go to: https://www.mobiledemand.com/drivers-downloads

  - Find your device model (1175)

  - Download the Power Delivery files

- Prepare the files

  - Find the file you downloaded

  - Right-click it and select Extract

- Prepare your USB flash drive

  - Plug in a USB flash drive (at least 1GB)

  - Right-click the USB drive and select Format

  - Choose FAT32 and click Start

- Copy the files to the USB drive

  - Open the USB drive

  - Copy the downloaded file (or extracted files) onto the USB drive

  - Make sure the files are not inside any folders

- Start the update

  - Plug the USB drive into your device

  - Turn the device on

  - Press the Delete (DEL) key repeatedly to enter settings

- Turn off Secure Boot

  - Go to the Security tab

  - Find Secure Boot

  - Change it to Disabled

  - Save and restart

- Boot from the USB drive

  - Press F11 while the device starts

  - Select the USB drive from the list

- Run the update

  - Wait for the screen to load

  - Follow the instructions on screen

  - Press any key when prompted to start the update

- Wait for completion

  - The update will take about 5 minutes

  - Do not turn off the device

- Finish up

- When you see the completion message, press any key to restart

- Turn Secure Boot back on (if needed)

- Press DEL to go back into settings

- Turn Secure Boot back to Enabled