Intel GNA Driver

Bring Up Guide

August 2018

Revision 0.5

Intel Confidential

INFORMATION IN THIS DOCUMENT IS PROVIDED IN CONNECTION WITH INTEL PRODUCTS.  NO LICENSE, EXPRESS OR IMPLIED, BY ESTOPPEL OR OTHERWISE, TO ANY INTELLECTUAL PROPERTY RIGHTS IS GRANTED BY THIS DOCUMENT.  EXCEPT AS PROVIDED IN INTEL’S TERMS AND CONDITIONS OF SALE FOR SUCH PRODUCTS, INTEL ASSUMES NO LIABILITY WHATSOEVER AND INTEL DISCLAIMS ANY EXPRESS OR IMPLIED WARRANTY, RELATING TO SALE AND/OR USE OF INTEL PRODUCTS INCLUDING LIABILITY OR WARRANTIES RELATING TO FITNESS FOR A PARTICULAR PURPOSE, MERCHANTABILITY, OR INFRINGEMENT OF ANY PATENT, COPYRIGHT OR OTHER INTELLECTUAL PROPERTY RIGHT.

A “Mission Critical Application” is any application in which failure of the Intel Product could result, directly or indirectly, in personal injury or death.  SHOULD YOU PURCHASE OR USE INTEL’S PRODUCTS FOR ANY SUCH MISSION CRITICAL APPLICATION, YOU SHALL INDEMNIFY AND HOLD INTEL AND ITS SUBSIDIARIES, SUBCONTRACTORS AND AFFILIATES, AND THE DIRECTORS, OFFICERS, AND EMPLOYEES OF EACH, HARMLESS AGAINST ALL CLAIMS COSTS, DAMAGES, AND EXPENSES AND REASONABLE ATTORNEYS’ FEES ARISING OUT OF, DIRECTLY OR INDIRECTLY, ANY CLAIM OF PRODUCT LIABILITY, PERSONAL INJURY, OR DEATH ARISING IN ANY WAY OUT OF SUCH MISSION CRITICAL APPLICATION, WHETHER OR NOT INTEL OR ITS SUBCONTRACTOR WAS NEGLIGENT IN THE DESIGN, MANUFACTURE, OR WARNING OF THE INTEL PRODUCT OR ANY OF ITS PARTS.

Intel may make changes to specifications and product descriptions at any time, without notice.  Designers must not rely on the absence or characteristics of any features or instructions marked “reserved” or “undefined”.  Intel reserves these for future definition and shall have no responsibility whatsoever for conflicts or incompatibilities arising from future changes to them.  The information here is subject to change without notice.  Do not finalize a design with this information.

The products described in this document may contain design defects or errors known as errata which may cause the product to deviate from published specifications.  Current characterized errata are available on request.

Contact your local Intel sales office or your distributor to obtain the latest specifications and before placing your product order.

Copies of documents which have an order number and are referenced in this document, or other Intel literature, may be obtained by calling 1-800-548-4725, or go to:  http://www.intel.com/design/literature.htm.

All products, computer systems, dates, and figures specified are preliminary based on current expectations, and are subject to change without notice.

This document contains information on products in the design phase of development. Do not finalize a design with this information. Revised information will be published when the product is available. Verify with your local sales office that you have the latest datasheet before finalizing a design.

Intel processor numbers are not a measure of performance. Processor numbers differentiate features within each processor family, not across different processor families. See www.intel.com/products/processor_number for details.

Code names featured are used internally within Intel to identify products that are in development and not yet publicly announced for release.  Customers, licensees and other third parties are not authorized by Intel to use code names in advertising, promotion or marketing of any product or services and any such use of Intel’s internal code names is at the sole risk of the user.

Intel, Core, Smart Sound Technology and the Intel logo are trademarks of Intel Corporation in the U.S. and/or other countries.

Microsoft, Windows, and the Windows logo are trademarks, or registered trademarks of Microsoft Corporation in the United States and/or other countries.

Bluetooth is a trademark owned by its proprietor and used by Intel Corporation under license.

*Other names and brands may be claimed as the property of others.

Copyright © 2018 Intel Corporation. All rights reserved.

Contents

1	Introduction	5

1.1	Purpose and Scope of Document	5

1.2	Acronyms and Terminology	5

1.3	What is needed for GMM Bring Up	5

2	GNA Bring Up	6

2.1	General Flow	6

2.2	GNA Support Platform	6

2.3	Enable GNA in BIOS	6

2.3.1	Cannon Lake and Ice Lake Platform	6

2.3.2	Gemini Lake Platform	7

2.4	Intel GNA Driver Installation	8

2.4.1	Manual Installation	8

2.4.2	Driver Installation Verification	9

2.4.3	Windows Update	9

2.5	Verify GNA Driver installation	10

3	GNA Functional Verification	14

3.1	Get the test environment ready	14

3.2	Automated verification test	14

Revision History

| Document Number | Revision Number | Description | Revision Date |
| --- | --- | --- | --- |
|  | 0.5 | Initial release. | August 2018 |

# Introduction

## Purpose and Scope of Document

This document provides installation instructions and bring up guideline to verify if the GNA device is available and functional on Intel platform with Intel GNA driver installed on Windows* 10 64bit Operating System.

All screenshots presented in the document are from Windows* 10 RS4.

## Acronyms and Terminology

| Term | Description |
| --- | --- |
| BIOS | Basic Input/output System |
| BKC | Best Known Configuration |
| CRB | Customer Reference Board |
| GMM | Gaussian Mixture Module |
| GNA | GMM and Neural Network Accelerator |
| IHV | Independent Hardware Vendor |
| ISV | Independent Software Vendor |
| MSFT | Microsoft Corporation |
| OS | Operating System |
| PCH | Platform Controller Hub |
| RVP | Reference Validation Platform |

## What is needed for GMM Bring Up

- GNA is enabled in platform BIOS

- Intel GNA driver installed in the system

- Files:  gna.sys, gna.inf, gna.cat

- The GNA self-test application set

- Files:  gan-self-test.bat, gna-self-test.exe, gna.dll

All files are distributed in Intel GNA release package.

# GNA Bring Up

## General Flow

- Check if GNA is available

- the platform supports GNA

- GNA is enabled in BIOS

- the GNA driver is installed correctly

If you cannot resolve the problem using the guide, contact the Intel CCE Audio Team

## GNA Support Platform

GNA functionality is available on the following Intel platforms:

| CPU Family Name | Device ID | Bus/Device/Function |
| --- | --- | --- |
| Gemini Lake (GLK) | PCI\VEN_8086&DEV_3190 | 0/3/0 |
| Cannon Lake (CNL) | PCI\VEN_8086&DEV_5A11 | 0/8/0 |
| Ice Lake (ICL) | PCI\VEN_8086&DEV_8A11 | 0/8/0 |
| Tiger Lake (TGL) | TBD | TBD |

## Enable GNA in BIOS

### Cannon Lake and Ice Lake Platform

- Enter platform BIOS set-up

- Check the GNA status at the following path (This path might differ on various platforms and BIOS versions):

- Intel Advanced Menu  System Agent (SA) Configuration  GNA Device (B0:D8:F0)   (Figure 1)

- If there’s no GNA entry, then

  - the BIOS version doesn’t provide with the GNA enabling capabilities  contact the Intel CCE team to get the correct BIOS version:

- Set ‘GNA device’ to Enabled

Figure 1 Cannon Lake Intel BIOS (example on an RVP board)

### Gemini Lake Platform

- Enter platform BIOS set-up

- Check the GNA status at the following path (this path might differ on various platforms and BIOS versions):

- Device Manager  System Setup  South Cluster Configuration  GNA Configuration  GNA Support (Figure 2)

- If there’s no GNA entry, then

  - the BIOS version doesn’t provide with the GNA enabling capabilities  contact the Intel CCE team to get the correct BIOS version:

- Set ‘GNA support’ to Enabled

Figure 2 Gemini Lake Intel BIOS (example on an RVP board)

## Intel GNA Driver Installation

### Manual Installation

The GNA driver can be installed manually using the INF file (see Figure 3):

  - click the right mouse button on “gna.inf”

  - select “Install”

  - expect message “The operation completed successfully” in the case of the successful installation

Fig.3. Manual GNA driver installation

### Driver Installation Verification

It is recommended to check in Windows* Device Manager if the newly installed GNA driver is seen:

  - go to “System devices” category and look for “Intel® GNA Scoring Accelerator module”

  - make sure there’s no new yellow bang

Refer to section 2.5 for more details.

### Windows Update

Windows Update can automatically install the GNA driver after the device detection if there’s no other GNA driver installed, including Null Driver.

The update of the GNA driver can be also enforced in Windows* Device Manager:

  - Find:

- “Base System Device” in the root directory

or

- “Intel® Celeron®/Pentium® Silver Processor Gaussian Mixture Model - 3190” in “System devices” category for the Gemini Lake platform

  - click the right mouse button on it

  - select “Update Driver”

- note: by default a newer drvier version overrides a previous one, however it is recommended to uninstall the previous GNA driver manually before installing the new one

  - choose option “Search automatically for updated driver software”   (see Figure 4)

If Windows* cannot install the GNA driver, refer to the sanity check in Chapter 2.

Figure 4 Windows Update: search automatically for updated driver software

## Verify GNA Driver installation

Open Windows* Device Manager

Go to “System devices” category and look for “Intel® GNA Scoring Accelerator module”

  - If the GNA device is present without Yellow bang (see Figure 5)

- Meaning:  GNA is available (supported by the platform, enabled in BIOS and the GNA driver installed correctly)

  - Bus, Device, Function and Device ID are listed in Properties (click the right mouse button and select “Properties”)

- Action:  go to the next step – GNA Verification (Chapter 3)

Figure 5 Correctly installed GNA driver (example)

Go to “Other devices” category and look for “Base System Device”

  - If the device is detected with the yellow bang (see  Figure 6)

- Meaning:  GNA is enabled in BIOS, and the GNA driver is missing

- Action:  install the GNA driver (Chapter 2.4)

Figure 6 Windows* Device Manager without GNA driver (example)

Go to “System devices” category and look for entries with “Gaussian Mixture Model”

  - If “Intel® Celeron®/Pentium® Silver Processor Gaussian Mixture Model - 3190” is present (see Figure 7)

- Meaning:  GNA is enabled in BIOS, and the Null Driver is installed

- Action:  upgrade to the correct GNA driver (Chapter 2.4)

Figure 7 Windows* Device Manager with Null Driver (example)

- If “Intel® Celeron®/Pentium® Silver Processor Gaussian Mixture Model - 3190” is NOT present (see Figure 7)

  - Meaning:  GNA is not detected, might have not been enabled in BIOS

  - Action: Enable GNA in BIOS (section 2.3)

# GNA Functional Verification

This chapter will describe how to verify GNA functionality.

- the GNA device is detected

- the GNA device functionally works

- the driver version is correct

In order to verify if GNA is up and running use the GNA self-test application distributed together with the GNA release package.

## Get the test environment ready

- Make sure the GNA driver is installed correctly (Section 2.4)

- Install “Redistributable Libraries for Intel® C++” for 64-bit Windows*

  - the libraries are available at:

- https://software.intel.com/sites/default/files/managed/a9/9c/ww_icl_redist_msi_2016.1.146.zip

- after having downloaded the libraries, unzip them and run the installation

- Install ”Visual C++ Redistributable for Visual Studio 2015”

  - the installer (vc_redist.x64.exe) is available at:

- https://www.microsoft.com/en-us/download/details.aspx?id=53840

- after having downloaded the installer, run the installation

## Automated verification test

- Run “gna-self-test.bat”

- Check the output message

  - if the message is “GNA self-test successful” (see Figure 8)

- meaning:  GNA is up and running

- action:  finish the sanity check

  - otherwise there’s a problem with GNA

Figure 8 Correct end expected result from GNA self-test application

- Resolve the GNA problem with respect to the error message:

  - if the message is “Detecting device... FAIL (no HW detected)”

- meaning:  the GNA device doesn’t exist  the platform does not support GNA

- action:  check the GNA availability on Intel platforms (section 2.5)

  - if the message is “Checking driver... FAIL (null driver installed)”

- meaning:  incorrect / null driver installed

- action:  follow the guidelines in section 2.5

  - if the message is “Checking driver... FAIL (no driver installed)”

- meaning:  no GNA driver is installed

- action:  install the GNA driver (Chapter 2.4)

  - if the message is “Checking driver... FAIL (GNA 2.0 driver installed instead of 1.0)”

- meaning:  wrong driver installed

- action:  upgrade the GNA driver to align its version to the GNA generation on the platform (Chapter 2.4)